Assignment 2

30 Marks

Deadline: Mar 10, 2026 (Tuesday)

## Question - 1:

Find the RISC-V code of the following machine Codes. [3.5 + 3.5] i. 0x00F29A93 ii. 0x089739B3

Format; Instruction; Opcode; Funct3; Funct7/Funct6. R; ADD; 0110011; 001; 0000000. SUB; 0110011; 110; 0000001. AND; 0110011; 010; 0000001. OR; 0110011; 011; 0000100. SLL; 0110011; 101; 0000100. I; LD; 0000011; 000; N/A. LW; 0000011; 010; N/A. LH; 0000011; 011; N/A. LB; 0000011; 001; N/A. ADDI; 0010011; 000; N/A. SLLI; 0010011; 001; 000000. SRLI; 0010011; 010; 000000. ORI; 0010011; 011; N/A. S; SD; 0100011; 000; N/A. SW; 0100011; 001; N/A. SH; 0100011; 010; N/A. SB; 0100011; 011; N/A.

## Question - 2:

while ( (a+13) > (c - 4) ):

if (a > b and b < c):

a = b

elif ( a == 5):

c = c + 10

else:

d = d + 10

Construct the equivalent RISC-V code of the above high-level code. Variables a,b,c,d are associated with register X11, X12, X13, X14 respectively [10]

## Question - 3:

Under which type(s) of instruction do Jalr and Store fall under? [1+1]

What is the significance of the funct3 field in case of a Load instruction?  [3]

## Question - 4:

| Memory Location | Code | Line Number |
| --- | --- | --- |
|  | Loop: |  |
|  | SLLI X10 , X22 , 3 |  |
|  | ADD X10 , X10 , X25 |  |
|  | LD X9 , 0(X10) |  |
|  | ADDI X17 , X0 , -7 |  |
|  | BNE X9 , X26 , Exit |  |
| #7048 | ADDI X22 , X22 , 1 |  |
|  | SD X10 , 16(X22) |  |
|  | BEQ X0 , X0 , Loop |  |
|  | Exit: |  |

Find the SB-type instructions from the above code and encode them accordingly. [4 + 4]

Given,

i) opcode = (103)10, funct3 = (011)2 opcode for BEQ.

ii) opcode = (103)10, funct3 = (010)2  opcode for BNE.

## Question - 5: [Ungraded]

int hishab(int i, int j) {

i = i + 8;

j = j - a[a [0] ];

while (j < i) {

j = a [i];

i = i + 1;

}

return j;

}

k = 2;

l = 3;

a [1] = hishab(k, l);

Translate the following codes written in high level language into an instruction sequence written in RISC-V Assembly. Assume that the variables k and l are in registers X5 and X6 respectively. a is a word array, which has its base address stored in X18. You need to use register X5 inside the function in a way so that it does not get overwritten so write your code accordingly.