Assignment 3

30 Marks

Deadline: Apr 28, 2026 (Tuesday)

## Question - 1:

Why is a bias added to the actual exponent in the IEEE 754 floating-point representation, and how does this affect the encoding of both positive and negative exponents? [5]

## Question - 2:

How does optimized multiplication improve efficiency and performance compared to traditional long multiplication, especially in terms of speed and computational complexity? [5]

## Question - 3:

Multiply (10110)2 with (101)2 using optimized multiplier, you are required to show the whole simulation. [5]

## Question - 4:

Perform the following multiplication using 24 bit IEEE-754 floating-point representation where the size of the fraction field is 14 bits. Check if the result has overflow or underflow. [5]

(-0.0101)2  x 279 and (10010.0101)2  x 287

## Question - 5:

Convert the following code to RISC-V assembly language. [10]

float a = fp_array [0];

float b = fp_array [1];

int c = 0;

if (a < b) {

c = (int) a;

} else {

c = (int) b;

}

Where fp_array is a floating point word array base address of which is stored in x13, a is in f14 and b is in f7.

Note: Assume that the numbers stored in fp_array are single precision floating point numbers.